# BPM Self Commissioning

# Description

The BPM self commissioning is a sw tool for automatic identification of BPM main parameters:

- Rs: stator phase resistance (Ohm)

- Ld: stator phase inductance on d-axis (d-axis = rotor flux axis) (H)

- Lq: stator phase inductance on q-axis (H)

- Phim: permanent magnet flux (V/rad/s)

- KTorque: the torque constant due to the permanent magnet (1.5*p*Phim)

- KDeflux: the torque constant due to the reluctance (1.5*p*(Ld-Lq))

The BPM self commissioning test is composed by three phases:

- High frequency voltage injection  Ld/Lq identification

- Dc voltage injection  Rs identification

- Bemf Observer speed close loop control  Phim, Ktorque, Kdeflux identifications

# HW Requirements

The communication with the board is performed through the microcontroller UART channel.

- A TTL – RS232 USB optoisolated converter is needed in order to connect the board to the PC running the Self Commissioning SW.

# SW Requirements

The autotesting is based on the following standard components:

- Python interpreter (3.6)  use the following guide to install Python: https://drive.google.com/open?id=1v-bfxefjiZIwiWRHGzR3U37f-EHdH-PMDYxP2-8Lhqo

- RTC, it is used as repository and version control.

and the following custom components:

- Dwarf_Parser: it is used to parse and handle elf files (used for the PIL approach)

- Master_Commander: it is used to manage communication between PC and the microcontroller using a light protocol (PIL approach)

# Setup for custom Fw

- Load “SELF_COMMISSIONING” component in your fw from RTC: _Debug_Testing_Lib stream in “Motor Control” Change Management

  - Load the Docs folder in Project structure docs folder:

  - Load the Source folder in Project structure source/Category/Common:

- Place the SelfCommissioning handlers in MCI callbacks (Mci_prm.h +  Mci_prv.h)

- Modifiy Motor parameters inside SelfCommissioning.c file- “Parameter_Initialize()”

  - Change motor pole pairs parameter according your application

- Compile and download your code.

# Setup Python for custom Fw

- Load the Python directory from “SELF_COMMISSIONING” component in the <workspace_root>/self_commissioning location;

- From the PYTHON_COMMUNICATION_LIB component load the master_commander folder in the Project <workspace_root>/self_commissioning/master_commander location;

- From the PYTHON_UTILITIES_LIB component load the dwarf_parser folder in the Project <workspace_root>/self_commissioning/dwarf_parser location;

- In the self_commissioning folder you should find a sample configuration file with cfg extension. Modify it according to your needs, following the comments in the file itself;

- Configure the COM parameters;

- Configure file paths parameters;

- Configure the measurements sections according to your application;

- Go to Run  Run Configurations and double click on Python Run;

- In the Dialog Window, select the current Project clicking the Browse button;

- Select the self_commissioning.py file as the Main Module;

- Select the Arguments tab. Choose other as the Working directory and browse the Workspace to select the self_commissioning directory;

- Provide the path to the configuration file in the Program arguments. If the file is in the same directory of the script (self_commissioning), just type ./name_of_config_file.cfg in there

- Click on the Apply button and then Run to launch the script.

- At the end of the procedure you should find the report in the self_commissioning/out folder.

- Click on the Apply button and then Run to launch the script.

- At the end of the procedure you should find the report in the self_commissioning/out folder.